Indicador de acción modelo Siemens FDAI93-EX, versión para montaje en marco de puerta, tableros de mando, techos o huecos. Para áreas con riesgo de explosión de zonas 1 y 2.

Made of transparent plastic, the FDAI93-Ex alarm indicator is suitable for installation in door frames, switching panels, ceiling voids or special housings. The 22.5 mm Ø mounting hole is drilled by the system owner. The holder must be fitted with the insulating cap for safety. For installation in humid rooms the DCA1191, DJZ1193 housings as well as a washer are available (must be ordered separately). Up to 4 alarm indicators can be built into the housing DCA1191. 

Technical details:
- Production category: IP56; IP65 with DJZ1193 
- Operating voltage: DC 1.5 V
- Operating temperature: -25 °C to +80 °C
- Operating current: Max. 20...30 mA (continuous)
- Ex-classification: Ex ib IIC T6 (Ta ≤ 55 °C) 'simple apparatus' acc. to EN 50020 § 5.4: Ex ib IIC T5 (Ta ≤ 70 °C)
- Dimensions: 26 x 51 mm
- Color: Transparent

| Product no. | Stock no. |  | Price |
| --- | --- | --- | --- |
| FDAI93-EX | S54370-F6-A1 |  | bajo pedido |