0

Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 CSI@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

BOSCH AUTODOME IP starlight 5100i IR (NDP-5533-Z30L 4MP outdoor IR PTZ camera)

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

BOSCH AUTODOME IP starlight 5100i IR (NDP-5533-Z30L 4MP outdoor IR PTZ camera)

- GENERAL

  - SUMMARY

    - Section includes a 4MP IP66-rated outdoor PTZ dome camera with integrated, long-life Illuminator, H.264 and H.265 encoding, (measured) 108 dB dynamic range, and rain-sensing wiper for improved images in rain and in dirty urban conditions.

## Related Requirements

      - 27 15 00 Communications Horizontal Cabling

      - 27 20 00 Data Communications

      - 28 05 00 Common Work Results for Electronic Safety and Security

      - 28 06 20 Schedules for Video Surveillance

      - 28 16 15 Access Control Interfaces to Video Surveillance

      - 28 23 00 Video Management System

      - 28 31 31.15 Intrusion Detection Interfaces to Video Surveillance

      - 28 51 15.13 Information Interfaces to Video Surveillance Systems

  - REFERENCES

    - Abbreviations

      - AI – Artificial Intelligence

      - ARP – Address Resolution Protocol

      - DHCP - Dynamic Host Configuration Protocol

      - DNS - Domain Name Server

      - DDNS – Dynamic DNS

      - DORI - Detect, Observe, Recognize, Identify

      - fps - frames per second

      - HEVC – High Efficiency Video Coding

      - HTTPS – Secure Hypertext Transfer Protocol

      - ICMP - Internet Control Message Protocol

      - IP - Internet Protocol

      - IR – InfraRed

      - JPEG - Joint Photographic Experts Group

      - mDNS – Multicast DNS

      - MP - Megapixel

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PKI - Public Key Infrastructure

      - PoE - Power over Ethernet

      - RTCP – Real-Time Control Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - TCP - Transmission Control Protocol

      - TPM - Trusted Platform Module

      - TLS – Transport Layer Security

      - UDP - User Datagram Protocol

    - Reference Standards

      - Network

        - IEEE 802.3 Ethernet Standards

      - Video

        - ITU H.264

        - H.265/HEVC

        - MJPEG

      - ONVIF

        - EN 50132-5-2

      - Emissions

        - FCC CFR 47 part 15 (Class A)

        - EN 55032 - Electromagnetic compatibility of multimedia equipment. Emission Requirements – Class A

        - EN 50130-4

        - EN 50121-4 (Railway applications)

      - Safety

        - EN 62368-1

        - IEC 62368-1

        - UL 62368-1

        - IEC 62471:2006

        - CAN/CSA-C22.2 No. 62368-1-19

      - Environmental

        - IEC

          - EN 60529 – Degrees of protection provided by enclosures – IP66

          - IEC 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts – IK10

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

      - Shop Drawings

      - Cyber hardening guidance

    - Closeout Submittals

      - Final listing of devices and settings

      - System test results

      - Statement of compliance with Manufacturer Cyber Hardening Guidelines

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The AUTODOME family of products is covered by a limited hardware warranty period of 3 years for moving parts and 5 years for non-moving parts, from the date of shipment, against any proved defect in materials or workmanship.

This warranty will be limited to a period of one year from the date of original purchase for moving parts such as, but not limited to:

      - Pan/Tilt drive and belts

      - Electrical slip-ring contacts

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 585 223 9180

Email: technical.support@us.bosch.com

    - Product Model:	NDP-5533-Z30L

  - GENERAL DESCRIPTION

    - The PTZ dome camera (“dome camera)” shall be a 4MP IP66-rated outdoor dome camera with integrated, long-life Illuminator, H.264 and H.265 encoding, (measured) 108 dB dynamic range, and rain-sensing wiper for improved images in rain and in dirty urban conditions.

    - The dome camera shall possess the following primary characteristics:

      - H.264, H.265, and MJPEG compression

      - Resolution up to 2560 x 1440 pixels

      - Frame rate up to 60 fps

      - Bi-directional audio

      - DORI technology to facilitate distinguishing of persons or objects

      - Multiple data security elements including Trusted Platform Module (TPM), PKI, and full end-to-end encryption.

      - Pre-configured optimized scene modes

      - Mobile device camera setup and configuration

      - IP66 and IK10 environmental ratings

      - Power:	24 VAC or PoE++ IEEE 802.3bt, Type 3 standard | PoE+ IEEE 802.3at Type 2 standard (no heater, half illuminator intensity

  - ADDITIONAL VIDEO/CAMERA SPECIFICATIONS

    - Imaging Device:

      - Size:		1/1.8 inch

      - Resolution:	4MP

    - Lens:

      - Focal Length: 			6.6 – 198 mm

      - Aperture:			F/1.5 – F/4.8

      - Horizontal Field of view:		2.1° x 58.5°

      - Lens zoom:			30x

      - Digital zoom:			16x

    - Video Streaming:

      - Compression type:		H.264, M-JPEG, H.265 HEVC

      - Resolutions:

        - 2560 x 1440

        - 1920 x 1080

        - 1280 x 1024

        - 1280 x 720

        - 768 x 432

        - 704 x 576

        - 640 x 480

        - 512 x 288

        - 256 x 144

      - Frame rate:			1 - 60 fps

      - Dynamic range:			108 dB (when measured according to IEC 62676 Part 5)

    - Night vision (Illumination):

      - Illumination type:		IR LED; white light

      - LED wavelength:		850 nm

      - Detection distance (IR):		320 m (1050 ft)

      - IR intensity:			90 lux

      - Beam angle:			55° FWHM

      - Detection distance (white light):	60 m (197 ft)

    - Additional Video Functions:

      - Day/night modes: Auto (adjustable switch points); Color; Monochrome

      - Backlight compensation

      - Video watermarking

      - Display stamping

      - Location

      - White balance modes:

        - Standard

        - Sodium lighting

        - Vibrant

        - LPR

      - Number of privacy masks:  32

      - Privacy mask patterns:

        - Black

        - White

        - Gray

        - Auto color

      - Minimum illumination (monochrome):	0.0016 lx

      - Minimum illumination (color):		0.0101 lx

      - Minimum illumination with IR:		0.0 lx

      - Video Content Analysis:

        - Object classes:

          - Person

          - Bike

          - Car

          - Truck

        - Object filters:

          - Duration

          - Size

          - Aspect ratio

          - Speed

          - Direction

          - Color

        - Alarm Triggers:

          - Any object

          - Object in field

          - Line crossing

          - Enter / leave field

          - Loitering

          - Follow route

          - Idle / Removed object

          - Counting

          - Occupancy

          - Crowd density estimation

          - Condition change

          - Similarity search

    - Local SD storage:	Micro SDHC or SDXC

    - Inputs and outputs

      - Alarm inputs:		2

      - Alarm outputs:		1

    - Audio

      - Line in

      - Line out

      - Compression:

        - G.711 8 kHz

        - L16 16 kHz

        - AAC-LC

      - Streaming:	Full duplex

    - ONVIF Conformity:

      - Profile S

      - Profile G

      - Profile T

      - Profile M

  - NETWORK

    - Connectivity: 10BASE-TX/100BASE-T Ethernet

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Internet Control Message Protocol (ICMP and ICMP v6), Hypertext Transfer Protocol (HTTP), Secure Hypertext Transfer Protocol (HTTPS)

      - Network services: Domain Name System (DNS), Dynamic DNS (DDNS), File Transfer Protocol (FTP), System Network Management Protocol (SNMP) v1, v3, MIBII

      - Media: Real-Time Control Protocol (RTCP), Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP); Internet Group Messaging Protocol (IGMP) v2/v3, DiffServ (QoS)

    - Data Security

      - Crypto coprocessor: 	AES/CBC 256 bit

      - Advanced handling certificates

          - Manufacturer origin device certificate pre-installed

          - Self-signed unique certificates automatically generated when required

          - Client and server certificates for authentication.

          - Client certificates for proof of authenticity.

          - Certificates with encrypted private keys.

      - End-to-end encryption: 	Full end-to-end with supported VMS

      - Encryption 		TLS 1.1; TLS 1.2; TLS 1.3; AES 256; AES 128

      - Video Authentication:	MD5; SHA-1; SHA-256

      - Local storage encryption: XTS-AES

      - Firmware protection:	Signed firmware, secure boot

  - ELECTRICAL

    - Power Input:

      - 24 VAC

      - PoE:	PoE++ IEEE 802.3bt, Type 3 standard | PoE+ IEEE 802.3at Type 2 standard (no heater, half illuminator intensity)

    - Power Consumption (maximum)

      - 24 VAC:	21.2 W – 39.4 W (typical)

      - PoE:		21.2 W – 39.4 W (typical – maximum)

  - MECHANICAL AND ENVIRONMENTAL

    - Construction Material: 	Aluminum

    - Finish options:		White

    - Impact Resistance:		IK10

    - Dimensions (Ø x H):  	285 mm x 456 mm (11 in. x 18 in.)

    - Temperature:

      - Operating: 	 		-40 °C to 60 °C (-40 °F to 140 °F)

      - Storage:			-40 °C to 60 ºC (-40 °F to 140 ºF)

    - Operating Relative Humidity:	5 to 93% (non-condensing)

    - Ingress Protection:		IP66

    - Mounting:

      - Pendant arm mount

      - Pipe mount

      - Pole mount

      - Pendant wall/ceiling mount

      - Roof mount

  - ACCESSORIES

    - The Manufacturer shall offer accessories to enable the mounting of the dome camera in the following circumstances:

      - Pendant – wall, pipe, pole, roof, or ceiling

    - Other Accessories

      - Mid-span injectors

      - 24 V power supplies

      - Spare bubble

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

  - STORAGE

    - Products shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

202503171656